DHS/CBP TRADE VPN

Deciding Which VPN Connection Type is Best for Your Company

Document Revision Level: 21 Revision Date: 12/30/2011

DHS/CBP Network Engineering Team Page 2 of 4

There are three scenarios that need to be considered when deciding which method to use to connect to the DHS/CBP Trade Partner VPN Infrastructure

Scenario #1: (Cisco IPSEC VPN Client)

Your company has high-speed Internet, will only need to send and retrieve data from one Windows 2000, XP, Vista or Windows 7 computer, and only needs to connect, briefly, to DHS/CBP a few times a day.

Connection Method:

LAN / DSL / Cable Modem based Internet connection

Platform Requirement:

- Windows 2000, XP, Vista or Windows 7

- If using a non-Windows platform, see scenario #2

Other Software Required:

- MQSeries Client **

- Cisco VPN Software Client version 4.8.01.0300 or later

- (DHS/CBP will provide with set-up instructions)

- Potentially $0 cost to establish connectivity to DHS/CBP

- Utilizes existing, high-speed Internet connection

Limitations:

- Cannot be used with MQSeries Server

- Connection needs to be manually initiated (can be scripted)

- Limited to “connect-put/get-disconnect” type connections

- Dropped connections will not auto-reconnect

- Connectivity to the WEB is blocked for that PC while it is connected to DHS/CBP

- Connectivity is limited to the locally connected LAN

Note: This scenario can not have a failover or extra client connections for testing!!

Scenario #2: (Internet based IPSEC LAN to LAN Connection)

Your company has high-speed Internet and will (one or more of the following):

- Desire the most robust VPN connection via the Internet

- Run MQ Client from a Windows or non-Windows 2000/XP/Vista/7 system

- Run MQ Server

Connection Method:

LAN / DSL / Cable Modem based Internet connection with Internet router with an available public IP address to dedicate to this connection

VPN Hardware Requirement (one of the following):

- Cisco 891 (Cisco mfg# CISCO891-K9 / estimated cost: $800)

- Cisco 1921 (Cisco mfg# CISCO1921-SEC/K9 / estimated cost: $1,000)

- Cisco 2911 (Cisco mfg# CISCO2911-SEC/K9 / estimated cost: $2,100)

- Cisco ASA 5505 - 5550 (Cisco mfg# ASA5505-50-BUN-K9 / estimated cost: $

cost: $500 ASA5550-50-BUN-K9 estimated cost: $12,000 )

DHS/CBP TRADE VPN

Deciding Which VPN Connection Type is Best for Your Company

Document Revision Level: 21 Revision Date: 12/30/2011

DHS/CBP Network Engineering Team Page 3 of 4

- NOTE: If your company would like to use a Cisco IPSEC device that is not listed above, please contact DHS/CBP’s Network Support Team at 1-877-347-1638, option 1, to determine if the device meets DHS/CBP’s requirements.

Other Software or Specification Required:

- MQSeries Client ** or Server

- Cisco IOS version level will be provided by DHS/CBP based on what is determined to be the most stable and secure code

Router configured with Cisco EZVPN mode is required for the Trade Partner configuration using MQSeries Client

- AES 256 IPSEC Encryption

Benefits:

- Most robust type of VPN connection

- No recurring monthly leased line charges

- Utilizes existing, high-speed Internet connection

- Tunnel creation, to and from DHS/CBP, can be initiated from either side (when using MQSeries Server)

- Data flows at time of creation, rather than going into a queue (when using MQSeries Server)

- Multiple systems can send data to DHS/CBP without additional software (when using MQSeries Server)

Limitations:

- Not controlled and monitored at the same service level as a dedicated MPLS connection to DHS/CBP (scenario #3)

Note: This scenario can not have a failover or extra client connections for testing!!

But if Server is being used a failover can be setup!!

Scenario #3: (MPLS based IPSEC LAN to LAN Connection)

Your company requires the assurance of a 24X7, always on, monitored connection with dedicated bandwidth and redundancy.

Connection Method:

- Dedicated and redundant MPLS* connections directly to DHS/CBP

VPN Hardware Requirement (one of the following):

- Cisco 1921 with WIC-1DSU-T1-V2 Bundle

o Mfg# CISCO1921-T1 / estimated cost: $1,300

- Cisco 2911 with WIC-1DSU-T1-V2

o Mfg# CISCO2911 / estimated cost: $2,200

- Mfg# WIC-1DSU-T1-V2= / estimated cost: $700

- NOTE: If your company would like to use a Cisco IPSEC device that is not listed above, please contact DHS/CBP’s Network Support Team at 1-877-347-1638, option 1, to determine if the device meets DHS/CBP’s requirements.

DHS/CBP TRADE VPN

Deciding Which VPN Connection Type is Best for Your Company

Document Revision Level: 21 Revision Date: 12/30/2011

DHS/CBP Network Engineering Team Page 4 of 4

- OPTIONAL: To accommodate Trade Partners that desire a backup connection for their dedicated circuit, use of an Internet VPN is supported as a secondary transport connection. This would require a separate VPN device selected from scenario #2 or #3.

Other Software or Specification Required:

- MQSeries Server

- Cisco IOS version level will be provided by DHS/CBP based on what is determined to be the most stable and secure code

- AES 256 IPSEC Encryption

Benefits:

- Most reliable type of connection

- Tunnel creation, to and from DHS/CBP, can be initiated from either side

- Data flows at time of creation, rather than going into a queue

- Multiple systems can send data to DHS/CBP without additional software

- Can be combined with Scenario #2 to enhance redundancy, in the event the MPLS connection goes down

Limitations:

- None

Details

- Contact Verizon or AT&T for costing.

- Pre-existing accounts – Contact your Verizon Business Account Representative or AT&T Account Representative for additional information.

- New accounts - Call Verizon 888-200-6002, option 1 and request support for service.

- New Accounts – AT&T – Browse website http://www.business.att.com/enterprise/Family/network-services/ip-vpn/?source=IEa225001ltpdsIP

* MPLS: Multiprotocol Label Switching – Termed “VBNS” by Verizon and “AVPN” by AT&T

** The automation of VPN Client and/or MQ Client put/get connections is permitted at an interval of no less than 5 minutes. In other words, please wait, or adjust your automated scripts to wait, at least 5 minutes between connections to DHS/CBP VPN/MQ.